Pir Testing Requirements 05-05-2024

All of the functions of the pir sensor must be tested and the results noted down and filed into the OneDrive folder

There need to be a technical data sheet made for the sensor complete with all of the details for the technical requirements including the compliance details and references.

There needs to be a user instruction sheet setting out all connections with explanation of function and connections

There need to be a web browser user and configuration instruction sheet

Board and function tests:

- Test for the input voltage lowest-highest and check for any sign of heat

- Test and note the device consumption in mv

- Test the leds and note the sequence, colour and functions are correct

- Test the dimming dips and there outputs in line with the settings and table below

| PIR Dimming Pin Setup and how it works |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Default setting |  | Pin setups |  |  |  |  |  |  |  |
|  | Max Output | 80% |  |  |  |  |  | Output | Difference |  |  |
|  | Dimming | 25% |  | 0 | 0 | 0 | 0 | 25.00% | 0.00% | Default |  |
|  |  |  |  | 0 | 0 | 0 | 1 | 28.67% | 3.67% |  |  |
|  |  |  |  | 0 | 0 | 1 | 0 | 32.33% | 3.67% |  |  |
|  |  |  |  | 0 | 0 | 1 | 1 | 36.00% | 3.67% |  |  |
|  |  |  |  | 0 | 1 | 0 | 0 | 39.67% | 3.67% |  |  |
|  |  |  |  | 0 | 1 | 0 | 1 | 43.33% | 3.67% |  |  |
|  |  |  |  | 0 | 1 | 1 | 0 | 47.00% | 3.67% |  |  |
|  |  |  |  | 0 | 1 | 1 | 1 | 50.67% | 3.67% |  |  |
|  |  |  |  | 1 | 0 | 0 | 0 | 54.33% | 3.67% |  |  |
|  |  |  |  | 1 | 0 | 0 | 1 | 58.00% | 3.67% |  |  |
|  |  |  |  | 1 | 0 | 1 | 0 | 61.67% | 3.67% |  |  |
|  |  |  |  | 1 | 0 | 1 | 1 | 65.33% | 3.67% |  |  |
|  |  |  |  | 1 | 1 | 0 | 0 | 69.00% | 3.67% |  |  |
|  |  |  |  | 1 | 1 | 0 | 1 | 72.67% | 3.67% |  |  |
|  |  |  |  | 1 | 1 | 1 | 0 | 76.33% | 3.67% |  |  |
|  |  |  |  | 1 | 1 | 1 | 1 | 80.00% | 3.67% | Max |  |
|  | Pin variant | 55% |  |  |  |  |  |  |  |  |  |

| Date | 04/03/2023 | 1-10 Dimming control Sensor Output Setup for WEB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Select Config value By user input cell |  |  |  |  |  |  |  |  |  |
|  | Dip Setup No. | 100 | 90 | 80 | 70 | 60 | 50 | 40 | 30 | 20 | 10 |
|  | 1 | 100% | 90.0% | 81.0% | 72.9% | 65.6% | 59.0% | 53.1% | 47.8% | 43.0% | 38.7% |
|  | 2 | 92% | 82.8% | 74.5% | 67.1% | 60.4% | 54.3% | 48.9% | 44.0% | 39.6% | 35.6% |
|  | 3 | 84% | 75.6% | 68.0% | 61.2% | 55.1% | 49.6% | 44.6% | 40.2% | 36.2% | 32.5% |
|  | 4 | 76% | 68.4% | 61.6% | 55.4% | 49.9% | 44.9% | 40.4% | 36.4% | 32.7% | 29.4% |
|  | 5 | 68% | 61.2% | 55.1% | 49.6% | 44.6% | 40.2% | 36.1% | 32.5% | 29.3% | 26.3% |
|  | 6 | 60% | 54.0% | 48.6% | 43.7% | 39.4% | 35.4% | 31.9% | 28.7% | 25.8% | 23.2% |
|  | 7 | 52% | 46.8% | 42.1% | 37.9% | 34.1% | 30.7% | 27.6% | 24.9% | 22.4% | 20.1% |
|  | 8 | 44% | 39.6% | 35.6% | 32.1% | 28.9% | 26.0% | 23.4% | 21.0% | 18.9% | 17.0% |
|  | 9 | 36% | 32.4% | 29.2% | 26.2% | 23.6% | 21.3% | 19.1% | 17.2% | 15.5% | 13.9% |
| Fixed Min output | 10 | 10% | 10% | 10% | 10% | 10% | 10% | 10% | 10% | 10% | 10% |

- Connect, Test and note the sensor connections from the pir and add a drawing to show connections from the sensor identifying the pinouts to the connections on the board and test in Realtime with a 1-10v driver and led light.

- Test the time out setting are working and relative to the dip setups and web configurations

- Test the sensitivity functions and note the results.

- Test the USB functions and note the results

- Test the wifi and mesh the three devices together and note the results and check the configuration is correct.

- Test the web application functions in remote and usb mode and note the results .

- Test the push functions and data string

Web App:

- Monitoring Page:

  - Make sure the data is displaying the correct information and that the memory is running in circular sequence so that the device cannot hang and the oldest data over writes first when its reaching the capacity, this should upload all the data once reconnection is established.

- Setup/System:

  - Set the device MAC id to Thermalimage one https://maclookup.app/search/vendors/result?vendor=thermalimage

Vendor

Details

OUI: 7C:83:34:0

Range: 7C:83:34:00:00:00 - 7C:83:34:0F:FF:FF

Block Size: 1048575 (1.04 M)

Universally administered addresses (UAA) : the address is uniquely assigned by its manufacturer.

Type of transmission: Unicast

  - General Setup and confirm it then applies and is attached into the sending string

  - Web Access configuration

  - Display settings title change to Light level and test it then applies and is attached into the sending string

  - Delay Settings title change to Dimming Timeout and test it then applies and is attached into the sending string

  - Sensitivity setup and check and test it then applies and is attached into the sending string

- System/WiFi:

  - STA mode configuration and functions.

  - AP Mode  configuration and functions.

  - Mesh functions

  - Hide the network name from being seen to the public tab

- Services/Push/NTP

  - Setup for http/https setup and push function and testing the data is sending in the string

  - Urls for to capture settings on the local machine and read to outgoing messages in parallel

  - NTP setup and auto scanning functions

- Administration:

  - User setups access and functions, check they store on the device memory and don’t get lost during a power loss and are auto recalled when repowered

  - Backup functions, check the original version of firmware is stored and the tab to factory reset works.

  - Check that configurations and settings store correctly and don’t get lost during a power loss and are auto recalled when repowered

  - Check that the firmware update setup works correctly and that the board can be updated OTA and reboots correctly and is fully accessible OTA after update